80MM Portable MINI Thermal Printer

                           Instruction Manual
(This manual applies to  80-II type version, and if you bought the machine to get the version, please print the self-test page title prevail through the machine)

Version 

Oct, 2014

Notice

• All rights reserved. Reproduction of any part of this manual in any form whatsoever, without manufacturer’s express permission is forbidden.

• The contents of this manual are subject to change without notice.

• All efforts have been made to ensure the accuracy of the contents of this manual at the time of going to press. However, should any errors be detected, manufacturer would greatly appreciate being informed of them.

• The above notwithstanding, manufacturer can assume no responsibility for any errors in this manual.

Declaration

We have been carefully handling the manual and trying our best to ensure the accuracy of the contents of this manual. But we still can’t promise it is always correct due to our products are being improved and updated all the time. The contents of this manual will be updated without notice at any time.

All of our products must be used under environments/locations which are taken care by human being. We are not responsible for any loss or damages caused by any accident while the products are used without taken-care by human being. If there are any products failures caused by human behaviors or natural disaster, we will not provide or just provide partial free maintenance.

Please contact local agents for the latest information of the products.

Notes on safety

This section presents important information intended to ensure safety and effective use of the printer. Please read carefully.

· Be sure to use the specified battery and power source provided by our company. Connection to an improper power source may cause fire , explosion or damage to the printer.

· Don’t put the battery into the fire or water, don’t disassemble or modify the battery, don’t be short circuit, otherwise may result in injury or fire even explosion.

· Please don’t open the paper room cover when it is printing or just when printing is over, do not touch the print head with hand or body, overheat may cause scald.
Notes on using

1、 Do not store the printer in hot and humid conditions.

2.、The lithium battery have a character that if it is not used for several months, it possibly can’t be charged ever. So for that, we suggest to use the adapter what less than 1A for charging .
3、The printer’s power adapter is to be connected to a stable power 110~220Volts, please do not use other devices on the same power socket, to avoid voltage fluctuations.

4、Avoid water or conductive material (for example: metal). If water is present on the surface please turn off the power immediately. 

5、Please ensure that a thermal paper roll is always inserted in the printer to avoid damaging the printing roller and the printer head.

6、Please ensure that your printer is serviced and repaired by a qualified technician. No responsibility is taken for misuse and/or non-qualified servicing.

7、In order to ensure print quality and longevity of your printer, please ensure that only good quality thermal paper rolls are used.

8、Please keep this manual in a safe place, where it can be accessed easily and used for your reference.
1.Features & Application area
一．Features 

· It can print 80mm width thermal paper

· Paper dimensions 40mm

· High-speed,Clear printing 

· Super power lithium electricity, standby time can reach 5-6 days

· Auto sleep, auto awake, save electricity

· small size、light weight、reliable performance、simple to connect、 easy to use

2． Application area  

Compared to the traditional thermal receipt printer, Mini printer has smaller body, more reliable capability, more steady printing and portable advantages. The Mini printer can work on a lot of locations, such as the TAXI bill printing, administrative fees receipt printing, post receipt printing, restaurant ordering information printing, online payment information printing, etc. With the rapid popularity of smart-mobile, Mini printer will be more widely used in the near future

2.Specification
Printing
Method: Direct Line Thermal

Speed:
70mm/sec

Reliability TPH Life: 100000000 pulse / dot or 100km

Resolution: 203DPI (8dot/mm)

Printing Width: 72mm (576dot)

Paper Width: 79.5±0.5mm

Paper thickness: 0.06-0.08 mm

Roll Diameter: φ≤40mm

Power: rechargeable lithium battery/NI-MH battery

Battery: 7.2VNiMh, 2000mAh

Charging method: external charger

Print parameters: support to download the Logo trademark

FontA:12*24dots,1.5(W)*3.0(H)mm

FontB: 9*17dots, 1.1(W)*2.1(H)mm

Simplified/Traditional:24*24dots,3.0(W)*3.0(H)mm

Printing Chinese character:  Chinese: simplified Chinese 24x24 point. support 18030 font library, support Taiwan and HK traditional(complex font)  ANK: ANK characters

Extended character table: 

OEM437/Katakana/OEM850/OEM860/OEM863/OEM865/West Europe/Greek/Hebrew/EastEurope/Iran/WPC1252/OEM866/OEM852/OEM858/IranII/Latvian/Arabic/PT1511251/OEM747/WPC1257/Vietnam/OEM864/Hebrew/WPC1255/Thai

Emulation: ESC/POS command

Line spacing: 3.75mm (Default)

Interface: Serial, USB, Bluetooth 2.0/Two-mode Bluetooth (optional), WIFI (optional)

Driver: Win200/Win2003/Win2000/WinXP /Win7/WIN8/LINUX/Android
Working Temperature: -10～50℃

Humidity: 10～90% 

Weight: 260g

Barcode Types: 

1D Barcode: UPC-A/UPC-E/JAN13(EAN13)/JAN8(EAN8)/CODE39/ITF/CODABAR/CODE93/CODE128

2D Barcode:QR CODE

3.Appearance


4.Communication Interface

·  1.  RS232(COM)：standard RS232 Communications Protocol 
·  2.  USB：standard USB Communications Protocol

·  3.  Bluetooth 2.0: General Bluetooth2.0 Communication Protocol

         Two-mode Bluetooth: General Bluetooth2.0 / 4.0 Communication Protocol

·  4.  WIFI: Standard 802.11b/g/n Protocol
4-1. RS-232 Serial Interface

RS-232 is developed according to the EIA standard asynchronous transmission, the specifications are as following:

Data transmission:  serial interface

Synchronization:    asynchronous

Signal Level:    RS-232 level, logic 1:-5.4v, logic 0 : +5.4v

Hardware Flow Control:   None

Baud rate:    2400pbs to 115200pbs is optional

Data word length:    8 bits

Stop bit:    1bit

Parity:  None

Users can get the current baud rate from the printer self-testing page. 

Serial cable: We have made the cable specially using mini port like Mini USB port which connects the printer .The following picture shows the signal definition of the RS-232 Serial interface: 

The connection between the printer and host follows the following rules:

	Printer
	Host

	TXD
	RXD

	RXD
	TXD

	GND
	GND


However, the printer usually works only on receiving data, it doesn’t have to respond anything back to the host , then TXD signal can be ignored , and then the connection can be sampled as following:

	Printer
	Host

	RXD
	TXD

	GND
	GND


4-2. Bluetooth Connection

Bluetooth is a proprietary open wireless technology standard for exchanging data over short distances (using short-wavelength radio transmissions in the ISM band from 2400–2480 MHz) from fixed and mobile devices, creating personal area networks (PANs) with high levels of security. 

4-2-1. Bluetooth specification

Using blueCore4-Ext chipset, fully compatible with Bluetooth 2.0; 

High-sensitivity receive signal, small size, lower consumption

Supporting 1200bps ~ 1382400bps variety of baud rate;

Supporting SPP agreement, the maximum transfer speed is up to 3Mbit / s;

4-2-2. Use “AT” command to modify Bluetooth Module’s parameters

Notice: The following changes can’t be done by user, these should be done in factory by Manufacturer. So if user wants to change these parameters, please inform our sales person before ordering our printers.

Modify name---Send character string “AT+NAMEXXXX, feedback “Okname”; 

For example, if you want change name to “LY58”, please send string “AT+NAMELY58”.

Modify PIN code--- Send character string “AT+PINXXXX, feedback “Oksetpin”;

For example, if you want change PIN code to “8888”, please send string”AT+PIN8888”.

(Note: Default PIN code is “1234”, you only can reset PIN code with four figures.)

Clear Bluetooth address--- Send character string “AT+CLR00”, feedback “Okclear”;

Modify baud rate--- Send character string “AT+BAUDX, feedback “Ok+current baud rate”; 

X represents the baud rate code, as following:

1-----1200                 7------57600

2-----2400                 8------115200

3-----4800                 9------230400

4-----9600                 A------460800

5-----19200                B------921600

6------38400               C------1382400

For example, if you want to change baud rate to 115200bps, please send string “AT+BAUD8”.

4-2-3. Build connection.

1. Turn on the printer;

2. Search for the Bluetooth device with host device;

3. If there are other Bluetooth devices, please select “LY58”(for example) printer;

4. Enter default pairing code “1234”

5. Pairing succeed.

4-3. USB Interface

The printer is connected by standard Mini USB cable(included in accessories) for communication. The USB type belongs to slave device type.
5.The installation of the paper roll
The printer use 80MM thermal paper, and adopt to easy paper loading. So the paper is very convenient install.  

Paper diameter Max 40mm.

Thermal paper installation method is as follows:

（1） Open the printer top cover 

（2） Thermal Paper as follow Graph to put into paper storehouse    [image: image1.emf]
（3）Put out a little of the paper outside printer,then close the top cover.

（4）Use paper sawtooth tear off redundant paper

Button Operating Instructions
	[image: image2.jpg]


（Green Light）
	ERROR(Red Light ）
	CHARGE(Red Light/Green Light)
	Printer Status

	OFF
	OFF
	OFF
	Printer Close

	Flash
	OFF
	OFF
	Printer dormant

	ON（Always）
	OFF
	OFF
	Printer open and have paper

	ON（Always）
	Flash
	OFF
	Printer Open , without paper

	
	
	CHARGE(Red Light)
	Charged with Battery

	
	
	CHARGE(Green Light)
	Full of Battery

	Display 1 or 2 shelf
	
	
	Low power of Battery

	Hold down power for 3 seconds
	Printer OPEN/OFF

	Under power on, the paper feeding begins when press 【FEED】 button
	the printer paper feed

	Power off the printer, then hold down the【FEED】button and press the power button at the same time. Wait 2 seconds ,then loosen the【FEED】button,the current setting will be printed out.
	The printer Self test 

	Power off the printer, then hold down the【FEED】button and press the power button at the same time. Wait until the power indicator &【ERROR】led are both on, and then 【ERROR】led goes off, then loosen the【FEED】button, the language setting status will be printed out.
	Transformation in both English and Chinese

	Power off the printer, then hold down the【FEED】button and press the power button at the same time. Wait until the power indicator &【ERROR】led are both on, and then 【ERROR】led goes out, after, 【ERROR】led goes on again ,then loosen the【FEED】button, the density setting status will be printed out. (It has three levels setting for printing density, it is a circle Low level(Middle level(High level(Low level…, Please switch the density status by repeating operation presented above.)
	Set printing density

	 Power off the printer, hold down the【MODE】button and press the power button at the same time. Wait until the power indicator &【ERROR】led are both on, and then 【ERROR】led goes out, then loosen the【MODE】button. The printer turns into the hexadecimal printing mode and print the presentation.Way to quit hexadecimal printing : The way to quit hexadecimal printing mode is switch off the printer .
	Hexadecimal printing 


6.The Bluetooth configuration

 Step1: Make sure your device (like PC, Note Book) has Bluetooth function, then once the Bluetooth for your device is activated, you will see the Bluetooth sign [image: image3.jpg]


 showed in the right bottom side of your device(like PC) screen.

[image: image4.jpg]EN





Step2:Power on the Bluetooth printer.

Step3:To add the Bluetooth printer, please left-click the [image: image5.jpg]


(click “Add a Bluetooth Device” ,then you will enter wizard screen , please choose “My device is set up according and ready to be found”(click “Next”(showed as below).

[image: image6.jpg]Professional

Add a Bluetooth Device
Show Bluetooth Devices

Send a File
Receive a File

Join a Personal Area Network

Open Bluetooth Settings

Remove Bluetooth Icon




[image: image7.jpg]Welcome to the Add Bluetooth
Device Wizard

Before proceeding, refer to the “Bluetooth™ section of the
device documentation. Then set up your device so that your
computer can find it:

- Tumiten
- Make it discoverable (visible)
- Give it a name [optional]

- Press the button on the bottom of the device
[keyboards and mice only]

() Add only Bluetooth d that you trust.

< Back

Cancel




Step4:if you can see screen as below,then the device is searching the bluetooth printer,please wait.

[image: image8.jpg]Add Bluetooth Device Wizard

Select the Bluetooth device that you want to add.

»

i i) Ifyou don't see the device that you want to add, make sure that it is
turned on. Follow the setup instructions that came with the device,
and then click Search Again.





Step5:Once the Bluetooth printer is found. You can see it listed on the screen, Here, for example, “BlueTooth Printer” is successfully found. Please click “BlueTooth Printer”,( click “Next”

Step6:Choose “Let me choose my own passkey”, then enter “1234”(this is factory default passkey, if you want to change it, you have to inform us to set the new passkey for you in our factory before shipping),then click “Next”.

[image: image9.jpg]Add Bluetooth Device Wizard

Do you need a passkey to add your device?

To answer this question, refer to the "Blustooth” section of the documentation that came with
your device. If the documentation specifies a passkey, use that one.

(O Choose a passkey for me

(O Use the passkey found in the documentation:

() Let me choose my own passkey: 1234

O Don't use a passkey

Q ‘You should always use a passkey, unless your device does not support one. We
recommend using a passkey that is 8 to 16 digits long. The longer the passkey, the
mare secure it will be.

<Back |[ Mew> | [ Cancel





Step7:if you can see the screen as below, that means the Bluetooth printer is already connected. Please notice the assigned COM ports ,then Click”Finish”.

[image: image10.jpg]Completing the Add Bluetooth
Device Wizard

The Bluetooth device was successhully connected to your
computer. Y our computer and the device can communicate
whenever they are near each other,

These are the COM [senall poets assigned to pour device,
@ofgoing COM pat COMS
Incoming COM post: COME

Leam moee about

To close this wizard, click Finish.

Finish





Step8: Now if you want to test if the printer is OK to print, you could use a com tool , and send printed data to Outgoing COM port( here, it is COM5).

(Com tool setting is 9600,N,8,1.)
7.The WIFI configuration

80mm MINI Portable Bluetooth Printer WIFI parameters configured through a Browser, the steps are as follows:

1. Plug and switch on the printer;

2. Find the wireless network and link it. Then find the file named “ HF-A11” and link it – Next ;
 3. Put the IP address ( 10.10.100.254 ) into the browser address bar, press enter we can see below window;

[image: image11.png]Connect to 10.10.100.254

i)

HE-ALL
User name:

Passuord:

€ advin v

[JRemember my password

==





Choose admin for the user name and default password is also admin. Press ok and then Next. 

[image: image12.png]) Mode Selection T ‘ Working Mode Configuration

5 AP Interface Setting
‘You may configure the Uart-WIFI module wifi mode 2nd data transfor mode.

5 STA Interfece Setting

) Application Setting

5 Device Management

Transparent lode





> Select “ Station Mode “, Choose transparent mode. Refer to above jpg. 

4. Select “ STA interface Setting ” as below;

[image: image13.png]£ Mode Selection

5 AP Interface Setting

8 Application Setting

&) Device Management

STA Interface Setting

‘You could configure STA interface parameters here.

AP'sSSID HF-AlLx_&F
MAC Address (Optional)
Security Mode OFEN v

Encryption Type TONE

WAN Connection Type: DHCF (duto_config) v

DHCP Mode

Hostname(Optional)




Then click “ Search”  and we can see and choose the wireless devices 
[image: image14.png]) Mode Selection STA Interface Setting

5 AP Interface Setting
‘You could configure STA interface parameters here.

5 STA Interfece Setting

8 Avplication Setting
s Device Management AP'sSSID HF-AlLx_&F
MAC Address (Optional)
Security Mode OFEN v

Encryption Type TONE

N2K ¥eb Server — Windows Internet Explorer

&) 1010 1025005 ] [ B] 49 [ X

ekl
Tipe

35D RSS |Channel | Encryption| Authentication

-
5:c5
& Ui _orst 20055976314 |345% |1 NONE | OPEN Infrastructure

HG2311-
O 2cE0 04:c0:6£2c:23'e8 | 24% | 1 NONE | OPEN Infrastructure

O Hroz sc2roatctendon [+ AES  |WPAZPSK |ifrastructure

20:dces:7a8e:aa] 65% |6 NONE | oPEN Infrastructure

© | chifateteTi | 00:1e:40tctar |24% |8 TKP |WPAPSK | Infrast




> Here is the wireless devices available for use.

5.  Choose the desired device as below

[image: image15.png]G > (B /710,10 100250751 9] [B] (%] [x] [@EE—T . ot [2]]
ss0 asoin s Cramnel Eneptn] Auneicaon S
P

(Bl orenra |005CEOTe3T94% 1 NONE |oPEN Infrastructure
HG23TE

[e] ety |04:c0:6£2c:23:e8 |29% | 1 INONE  |OPEN Infrastructure

O |Hv-o02 8c21:0a1c19:34 0% [4 aES WPAZPSK | Infrastructure

O |chinaNeteTit | 00:1e:4004:chaf [29% 5 TKIP WPAPSK | Infrastructure

ol 20:dce6:7a8e:aa 65% |6 NONE  |oPEN Infrastructure.

O |mrwang 02223fbsaefo 0% |8 AES WPAZPSK | Infrastructure
Apply





          > Click “ Apply” 

 6. Enter the correct configuration in the dialogue as below jpg shows.

[image: image16.png]) Mode Selection STA Interface Setting

5 AP Interface Setting

‘You could configure STA interface parameters here.

5 STA Interfece Setting

& Application Setting STA nterta

s Device Management AP'sSSID TP-LINK_97E314
MAC Address (Optional)
Security Mode
Encryption Type

Pass Phrase

WAN Connection Type: DHCF (duto_config) v

DHCP b

Hostname(Optional)




[image: image17.png]£ Mode Selection

5 AP Interface Setting
5 STA Interfece Setting
8 Application Setting

&) Device Management

STA Interface Setting

‘You could configure STA interface parameters here.

apsssm Te-LINE 67314
MAC Address (Optional)

Security Mode VPAZPSK v

Encryption Type P v

Pass Phrase —

WAN Connection Type:

TIC(fixed TP) v

DCF (huto_config)

TP Address 0.0.0.0
Subnet Mask 0.0.0.0
Default Gateway 0.0.0.0




[image: image18.png]£ Mode Selection

5 AP Interface Setting
5 STA Interfece Setting
8 Application Setting

&) Device Management

STA Interface Setting

‘You could configure STA interface parameters here.

apsssiD TP-LIE 7E314

MAC Address (Optional)

Security Mode WPAZPSK v
Encryption Type TKIP v
Pass Phrase .

WAN Connection Type: STATIC (fixed TP) v

192. 165. 1. 105
Subnet Mask 255. 256. 256. 0
192, 168. 1. 1]

[ -




           > Click “ Apply ” after all these configuration steps ;

7.Select “ Serial and other settings ” as below;

[image: image19.jpg]@

5 Device Management

Uart Sefiing
Data Bits sy

Parity None S The Baudrate choose 9600
Stop Tv

CTSRTS Disable v

UART A

UART AutoFrame Disable v

Protocol ¢V

Server Address

MAX TCP Num. (1-32) 32

TCP Time out (MAX 600 s) 300




>  Click “Apply” after all setting done.
 8. Restart the module as below;

[image: image20.png]) Mode Selection Device Management

£ AP Interface Setting ors
sy STA Interface Setting o ey configureadminsrator sceeunt and passord d dflt seig o udate
& Application Setting e
£ Device Mansgement

Adminsirat

Account adnin

Password adnin

art Module

Restart Module

Load Default Button Load Default
Update Firmware

Location:





[image: image21.png]ng Web Server

Rebooting...





> Module configuration done
9 . Computer “Start”       “ Run” Input word “CMD”，we have below dialog：
[image: image22.png]5.1.26001
<G> FRATHTE 1985-2681 Microsort Gorp-

:\Docunents and Settings\Adninistratord.





> Input “ping 192.168.1.105 -t” as below 

[image: image23.png]<G> FRATHTE 1985-2681 Microsort Gorp-

:\Docunents and Settings\Adninistrator>ping 192.168.1.185 —





> Press “Enter”

If details shows as below , means connection is Ok ;

[image: image24.png]oraseft Vindos XF [T 5.1.26001 =
o> FRATHTE 1985-2681 Microsort Gorp-

:\Docunents and Settings\Adninistrator>ying 192.168

105
[Pinging 192.168.1.105 with 32 hytes of data:

[Reply from 192.168.1.165: hytes=32 time=6ims TTL=255
[Reply £rom 192.168.1.185:
[Rep1y £rom 192.168.1.185
[Rep1y £rom 192.168.1.185
[Reply £rom 192.168.1.185:
[Rep1y £rom 192.168.1.185
[Rep1y £rom 192.168.1.185
[Reply from 192.168.1.105:





8.Driver Install

The first thing is to connect the usb cable between the printer and the computer,and power on the printer.
The second thing is to find the printer driver named “setup.exe” included in the CD,and double-click on it. Then we are entering the installing screen. Follow the steps in order:

Step1:select the language you want. We recommend that choose “English” as the language. Then click on “OK” to continue.

[image: image25.png]Select Setup Language

Select the language to uss during the.
installaton:





Step2:click on “Next” to continue.

[image: image26.png]HEE]

Welcome to the POS80 Series
Printer Driver Setup Wizard

This il nstall POSED Series Printer Drver version 1.5 an your
computer.

It s recommended that you dose all other applcations before:
cantinuing,

Click Nt to continue, or Cancel to exit Setup.





Step3:Select the agreement option as below, and click on “Next” to continue.

[image: image27.png]License Agreement
Please read the follwing impartant nformation before cantinuing,

Please read the following License Agreement, You must accept the terms of this
sgreement before continuing with the nstalaton.

[SOFTWARE LICENSE AGREEMENT

IMPORTANT - PLEASE READ CAREFLLLY! The POSBD Series Printer Drver
Jsaftware you are about to download wilbe licensed to yau, the liense, on the.
lcanition that you agree with the terms and condtions set Forth n this legal
lsgreement. PLEASE READ THIS AGREEMENT CAREFLLLY. YOU WILL BE BOUND.
/B THE TERMS OF THIS AGREEMENT IF YOUI INSTALL, DOWNLOAD, COPY, OR
(OTHERWISE USE THE SOFTWARE. I you do not agree to the terms contained in
ths agreement, plsase do not nstallor dovriad the softwere. Plesse record
the date of donrload in rder to activate the Imited rinety (30) day warranty
(see below).

(@)1 accept the agreement
Tdo ot accept the agreement

<Back Next > Cancel





Step4:Click on “install” to continue.

[image: image28.png]72 Setup - POSBO Series Printer Driver

Ready to Install

Setup i now ready to begin nstaling POSBO Series Printer Driver on your

computer.

ClickInstall o continue vith the instaltian.

Ce 1





Step5:Select the Operating System you are using,then click on “Next” to continue.

[image: image29.png]72 Setup - POSBO Series Printer Driver

7, Select Operating System

Select Dperating Systern

© Windows 2000 WindowsXP " Windows 2003

© WindowsVista © Windows7 " Windows 8





Step6: In this setting screen, you just have to set option

“Printer Port Settings”.

Ports: choose “USB001”,and then click “OK” to continue.

[image: image30.png]Printer Type:[F0580 -

- Pinter selfings

¥ Set as defaul prrter

Printer Name:  [F500

- Pinter Port Settings

Fots ) -

Badae [0 =] swebis [1 =] FiowConvol [T
Beste [ ] Pav  [or o
Back o Cance





Step7:Just click on “Continue Anyway” to continue.

[image: image31.png]Completing the POS80 Series
Printer Driver Setup Wizard

Setup has firished nstaling POSB0 Series Printer Drtver on
your computer.

Click Finsh to extt etup.

[ view the printer's property sheet.





Bluetooth port choose：

Select COM port what automatically assigned by Bluetooth Device in the ‘PORT”, Bluetooth serial port settings and corresponding baud (factory default Bluetooth baud rate: 9600, N, 8,1,), in the "Device Settings"- "Print" is selected for "soft fonts".
WIFI port choose：

1,Open the Printer Properties screen, then select the "Port" item, as shown below:
[image: image32.jpg]==
TR J

2 US Vi gt ot
01 S8 Ve porves pot .

(IJ_:I

)t srves ok





1. Enter the IP address into “Printer Name” dialog, and click “Next” to continue the installation.

[image: image33.png]2

Fle Edt View Favortes Took Help

Do | D sewn [ ok ||
Address [©, printers and Faxes

A .

Add Port
For which device do you wart to add 3 port?

Ente the Pinter Name o1 [P address, and a pot name for the desied device.

Priter Name or P Address: | 192168123100

Port Name: 1P_152168123.100

CBack_J_New> Corcel

SRY ro1pm





2. Click "Port" to Select “Add Standard TCP/IP Printer Port Wizard”, click “Next” to continue the installation.

[image: image34.png]2
Flo Edt View Favortes Tor

ok Help

0~ © B

e [ rodrs ||

Address [2, printers and Faxes

Welcome to the Add Standard
TCP/IP Printer Port Wizard

Yo uss this wizaid o add a port or 3 network piiter.

Belore continuing be sure that
1. The device is tmed an.
2 The network is connected and configured.

To continue, cick Nex.

RSB 100





3. Add the standard TCP / IP Printer Port Wizard, select the device type, and then click “Next” to continue the installation, See the following image:

[image: image35.png]S
2
Fle Edt View Favortes Took Help | o

Do | D sewn [ ok ||
Address [©, printers and Faxes T

A .

Additional Port Information Required
The device could nt be idertiied s

The device i nat found an the netwark, Be sure that

1. The device s tumed on

2 The network is connected.

3. The device s properly configured.

4. The addess on the previous page i cortect

1 youthink the s s not corect,click Back to etun o the previous page. Then cortect
the address and perform anather seaich on the network. |f you are sure the address is corect,
selectthe device type belon.

Standard v

Otustom e





4. Click “Finish” to complete the installation, see the following image:

[image: image36.png]S .
> HEE]

Fle Edt View Favorites Tools Help | o

Do | D sewn [ ok ||
Address [©, printers and Faxes

Completing the Add Standard
TCP/IP Printer Port Wizard

Yo have selected a port with the following cheracterisics.

SHMP: No
Protocol  RAW, Port 9100
Devies 192168123100
FoitName:  IP_192168.123.100
Adopter Type: Generic Network Card

To completethis wizard,click Firish.

Bk J[_Frn Concel
Py A
Y e RY tozem





5. Installation will return to “Properties” interface, and then select the correct IP port, click “Apply”, you can now print a test page. See the following image:
[image: image37.png]e —

Fle Edt View Favortes Took Help

Dot O | Psewen [ s | S <]

Gererd Shaing Pots | dvanced| Devi Saings
Address 2, printers and Faxes | 2 I I =
A O s é stpen

— .
=1

|| Bt T e e e et e

Sl s

Pot_ | Deserption Pintr
0 come: SeiPor
O comt: SeiaPor
[2) Troubleshoot printing O FILE: PrinttoFile
@ Gethep it g 0 USe... Vinslpiter o
01 USB..._Vitual prnte port fo
1P Stondad TCPAP Por

See Also

»)

Add Por. Delete Part Configure Port

Enable bidiectionl suppart
[ Enable pinter pooiing

Other Places

B Controlanel
S Scanners and Cameras

Close. Cancel Apply





[image: image38.png]5 POS0 10007 Device Sttings
) Form To Tray Assignment

Manual Paper Feed: ZPrinter Paper(80x 276mm)
&8 Instalable Optons

PrintSpeedt: Fast mode

Print Mod: | T

Cosh Drawer: Prin o mage

Print method[ 77Ty

Blank space at page’s &

Feed distance ate print: feed 0

Paper Cutting:After one page

Beeper Beep before every page

NV Logo: Do not print NV Logo

Character Set PCA37(Std Europell)





> Return to “ General ” and click “ Print Test Page ” as below.

[image: image39.png]Colon No
Double-sided: No.
Staple:No

Speed: Unknown

Model: POSE010007

Paper available:

Zprinter Paper(®0 x 3276mm)

Maximurn resolution: 203 dpi





> Everything is ok if the test page print out in good condition.
9.Cleaning of printer head

Cleaning of printer

When the printer has the following conditions, you should clean the printer head:

1　 The printer does not print clearly and the thermal paper is Ok.

2　 The printer Print pages lengthways is not clear

3　 It’s getting too noisy while feeding paper or printing.

Printer head cleaning steps are as follows:

 1. Power off the printer and open the paper room cover. Remove the thermal paper roll.

 2. If the printer just finished printing , please wait for the print head to cool down.

 3. With a soft cotton cloth dipped in ethanol (please make sure there is no dripping), wipe the thermal printer head gently and remove any dust or stains.

 4. Wait for the complete evaporation of any ethanol, then place the thermal paper roll back in the printer and close the paper room cover, then print a test page.

Notes：

· When the printer maintenance must ensure that the power is shut down

·  Hands or other metal tools away from the surface of the printer head,do not use tweezers to scratch the surface of the printer head and other sensitive parts.

· Do not use gasoline,acetone and other organic solvents for printer head cleaning.

10.General Troubleshooting
1: Why serial port printer printing messy code?
A: Ensure the baud rate setting in the PC is same as the printer baud rate. ( we can see printer baud rate from self test page)
2: Self printing test is ok,but printer does not work after driver installation.
A: Recheck the installation according to above installation wizard and see if we selected the right port or ask service center for advice if needed.
3: Malfunction of paper feeding?
A: Use soft brush to clean the paper sensor and try again or send it to the service center for repair.
4．Malfunction of the indicator light
A: Check the cable connecting way / adapter / power cord / switch on or off? Or contact the service center for repair.
5．Messy code printing except Arabic numeral printing.
A: language switching incorrectly,please turn on the SW-4 switch on the DIP switches.
6．Red indicator keep flashing during the printing.

A: Usually it is due to the high temperature of the print head. If printing stopped,please contact the service center for repair.

7. Printing speed is very slow for serial printer 

A: follow this path and change the set accordingly : Driver -- Properties -- Device Settings -- Print Mode -- Print as soft font.

       8. Water or other liquid spills into the printer
       A: Cut off the power,we can dry the main board or printer head with a hair dryer, careful of the temperature ( average 50 degree is ok )
9. The printer can't open 

       A：The battery maybe without electricity，please use power adapter to charge. Then you try to open the printer, if the printer has same problem, please send to service center for repair.     

       10. Do not search to Bluetooth

       A：Please check the main equipment "Bluetooth" whether opened, the communication whether normal, then plug the printer "Bluetooth Module" again. the printer re-connect, if there are problems, please send the specified location repair. 
Size：103*100*46 mm


Weight：260g(With Battery)











